/*

AGRI LINK - ESP32 Smart Agriculture Display

OLED: SSD1306 128x64, software SPI

Pages rotate every 5 seconds; alerts override normal pages.

Libraries:

Adafruit GFX, Adafruit SSD1306, DHT sensor library,

VL53L0X (Pololu), TinyGPSPlus

Pins are based on the user's existing wiring.

*/

#include <Wire.h>

#include <SPI.h>

#include <math.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#include <DHT.h>

#include <VL53L0X.h>

#include <TinyGPS++.h>

// OLED software SPI pins

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

#define OLED_MOSI 23

#define OLED_CLK 18

#define OLED_DC 19

#define OLED_CS 5

#define OLED_RESET 4

Adafruit_SSD1306 display(

SCREEN_WIDTH, SCREEN_HEIGHT,

OLED_MOSI, OLED_CLK, OLED_DC,

OLED_RESET, OLED_CS

);

// Sensor / output pins

#define DHT_PIN 27

#define DHT_TYPE DHT11

#define LDR_PIN 35

#define SOIL_PIN 34

#define GPS_RX 16

#define GPS_TX 17

#define BUZZER_PIN 32

#define TRIG_PIN 12

#define ECHO_PIN 14

#define LED_RED 25

#define LED_BLUE 26

#define LED_GREEN 33

DHT dht(DHT_PIN, DHT_TYPE);

VL53L0X laser;

TinyGPSPlus gps;

HardwareSerial GPS_Serial(2);

// Timing

const unsigned long PAGE_INTERVAL_MS = 5000;

const unsigned long FRAME_INTERVAL_MS = 100;

const unsigned long SENSOR_INTERVAL_MS = 300;

const unsigned long DHT_INTERVAL_MS = 2000;

// Alert thresholds (adjust after testing)

const float ULTRASONIC_NEAR_CM = 10.0;

const uint16_t LASER_NEAR_MM = 200;

const float HIGH_TEMP_C = 40.0;

// Soil alert is disabled by default until the raw sensor is calibrated.

const bool ENABLE_SOIL_ALERT = false;

const int SOIL_DRY_THRESHOLD = 3000;

// Calibrate these using your own dry and wet soil readings.

// Typical capacitive sensors often read lower when wet, but modules vary.

const int SOIL_DRY_RAW = 4095;

const int SOIL_WET_RAW = 1500;

// Live sensor data

float temperatureC = NAN;

float humidityPct = NAN;

float ultrasonicCm = -1.0;

int soilRaw = 0;

int ldrRaw = 0;

uint16_t laserMm = 0;

bool laserReady = false;

bool gpsFix = false;

double latitude = 0.0;

double longitude = 0.0;

// Pages: 0 welcome, 1 temp, 2 humidity, 3 soil, 4 light,

//        5 laser, 6 ultrasonic, 7 GPS, 8 animated farm

const int PAGE_COUNT = 9;

int currentPage = 0;

int animationFrame = 0;

int activeAlert = 0; // 0 none, 1 ultrasonic, 2 laser, 3 temp, 4 soil

unsigned long lastPageChange = 0;

unsigned long lastFrame = 0;

unsigned long lastSensorRead = 0;

unsigned long lastDhtRead = 0;

// ---------- Small drawing helpers ----------

void drawCenteredText(const char *text, int y, uint8_t size) {

display.setTextSize(size);

display.setTextColor(SSD1306_WHITE);

int16_t x1, y1;

uint16_t w, h;

display.getTextBounds(text, 0, y, &x1, &y1, &w, &h);

display.setCursor((SCREEN_WIDTH - (int)w) / 2, y);

display.print(text);

}

void drawHeader(const char *title) {

display.setTextSize(1);

display.setTextColor(SSD1306_WHITE);

display.setCursor(0, 0);

display.print(title);

display.drawLine(0, 10, 127, 10, SSD1306_WHITE);

}

void drawFooter() {

display.drawLine(0, 56, 127, 56, SSD1306_WHITE);

display.setTextSize(1);

display.setTextColor(SSD1306_WHITE);

display.setCursor(0, 57);

display.print("AGRI LINK");

display.setCursor(99, 57);

display.print(currentPage + 1);

display.print("/");

display.print(PAGE_COUNT);

}

void drawSun(int x, int y, int r) {

display.drawCircle(x, y, r, SSD1306_WHITE);

for (int i = 0; i < 8; i++) {

float a = i * 0.785398f;

int x1 = x + (int)(cosf(a) * (r + 2));

int y1 = y + (int)(sinf(a) * (r + 2));

int x2 = x + (int)(cosf(a) * (r + 5));

int y2 = y + (int)(sinf(a) * (r + 5));

display.drawLine(x1, y1, x2, y2, SSD1306_WHITE);

}

}

void drawWaterDrop(int x, int y, int r) {

display.drawCircle(x, y, r, SSD1306_WHITE);

display.drawTriangle(x, y-r-4, x-r, y, x+r, y, SSD1306_WHITE);

}

void drawPlant(int x, int groundY, int height) {

display.drawLine(x, groundY, x, groundY-height, SSD1306_WHITE);

display.drawLine(x, groundY-height/2, x-5, groundY-height/2-4, SSD1306_WHITE);

display.drawLine(x, groundY-height+3, x+5, groundY-height+7, SSD1306_WHITE);

}

void drawTree(int x, int groundY) {

display.drawLine(x, groundY, x, groundY-10, SSD1306_WHITE);

display.drawCircle(x, groundY-15, 6, SSD1306_WHITE);

display.drawCircle(x-4, groundY-12, 4, SSD1306_WHITE);

display.drawCircle(x+4, groundY-12, 4, SSD1306_WHITE);

}

void drawButterfly(int x, int y, bool wingsOpen) {

if (wingsOpen) {

display.drawCircle(x-3, y-2, 3, SSD1306_WHITE);

display.drawCircle(x+3, y-2, 3, SSD1306_WHITE);

} else {

display.drawLine(x-4, y-2, x, y, SSD1306_WHITE);

display.drawLine(x+4, y-2, x, y, SSD1306_WHITE);

}

display.drawLine(x, y-2, x, y+3, SSD1306_WHITE);

}

void drawFarmer(int x, int y, bool walkingStep) {

display.drawCircle(x, y, 3, SSD1306_WHITE); // head

display.drawLine(x-4, y-3, x+4, y-3, SSD1306_WHITE); // hat

display.drawLine(x, y+3, x, y+10, SSD1306_WHITE); // body

if (walkingStep) {

display.drawLine(x, y+5, x-5, y+7, SSD1306_WHITE);

display.drawLine(x, y+5, x+5, y+2, SSD1306_WHITE);

display.drawLine(x, y+10, x-5, y+15, SSD1306_WHITE);

display.drawLine(x, y+10, x+4, y+13, SSD1306_WHITE);

} else {

display.drawLine(x, y+5, x-4, y+8, SSD1306_WHITE);

display.drawLine(x, y+5, x+4, y+8, SSD1306_WHITE);

display.drawLine(x, y+10, x-3, y+15, SSD1306_WHITE);

display.drawLine(x, y+10, x+3, y+15, SSD1306_WHITE);

}

}

void drawChicken(int x, int y) {

display.drawCircle(x, y, 4, SSD1306_WHITE);

display.drawCircle(x+4, y-4, 3, SSD1306_WHITE);

display.drawLine(x+7, y-4, x+10, y-3, SSD1306_WHITE);

display.drawLine(x-2, y+4, x-3, y+7, SSD1306_WHITE);

display.drawLine(x+2, y+4, x+3, y+7, SSD1306_WHITE);

}

void drawCow(int x, int y) {

display.drawRoundRect(x, y-8, 15, 8, 2, SSD1306_WHITE);

display.drawCircle(x+17, y-7, 4, SSD1306_WHITE);

display.drawLine(x+3, y, x+3, y+3, SSD1306_WHITE);

display.drawLine(x+11, y, x+11, y+3, SSD1306_WHITE);

display.drawPixel(x+7, y-5, SSD1306_WHITE);

}

// Pseudo-3D perspective crop rows

void drawPerspectiveField() {

display.drawLine(0, 33, 127, 33, SSD1306_WHITE);

display.drawLine(0, 55, 48, 33, SSD1306_WHITE);

display.drawLine(32, 55, 54, 33, SSD1306_WHITE);

display.drawLine(64, 55, 64, 33, SSD1306_WHITE);

display.drawLine(96, 55, 74, 33, SSD1306_WHITE);

display.drawLine(127, 55, 80, 33, SSD1306_WHITE);

for (int y = 38; y <= 53; y += 5) {

int halfWidth = map(y, 33, 55, 0, 63);

display.drawLine(64-halfWidth, y, 64+halfWidth, y, SSD1306_WHITE);

}

drawPlant(12, 48, 5);

drawPlant(28, 51, 6);

drawPlant(46, 46, 5);

drawPlant(82, 46, 5);

drawPlant(100, 51, 6);

drawPlant(116, 48, 5);

}

// ---------- Read sensors ----------

float readUltrasonicCm() {

digitalWrite(TRIG_PIN, LOW);

delayMicroseconds(2);

digitalWrite(TRIG_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);

unsigned long duration = pulseIn(ECHO_PIN, HIGH, 25000UL);

if (duration == 0) return -1.0;

return duration * 0.0343f / 2.0f;

}

void updateGPS() {

while (GPS_Serial.available() > 0) {

gps.encode(GPS_Serial.read());

}

gpsFix = gps.location.isValid() && gps.location.age() < 5000;

if (gpsFix) {

latitude = gps.location.lat();

longitude = gps.location.lng();

}

}

void readSensors() {

unsigned long now = millis();

updateGPS();

if (now - lastSensorRead >= SENSOR_INTERVAL_MS) {

lastSensorRead = now;

soilRaw = analogRead(SOIL_PIN);

ldrRaw = analogRead(LDR_PIN);

ultrasonicCm = readUltrasonicCm();

if (laserReady) {

uint16_t reading = laser.readRangeContinuousMillimeters();

if (!laser.timeoutOccurred()) laserMm = reading;

else laserMm = 0;

}

}

if (now - lastDhtRead >= DHT_INTERVAL_MS) {

lastDhtRead = now;

float t = dht.readTemperature();

float h = dht.readHumidity();

if (!isnan(t)) temperatureC = t;

if (!isnan(h)) humidityPct = h;

}

}

int calculateSoilPercent() {

if (SOIL_DRY_RAW == SOIL_WET_RAW) return 0;

int p = map(soilRaw, SOIL_DRY_RAW, SOIL_WET_RAW, 0, 100);

return constrain(p, 0, 100);

}

// ---------- Alert priority ----------

int determineAlert() {

if (ultrasonicCm > 0 && ultrasonicCm <= ULTRASONIC_NEAR_CM) return 1;

if (laserReady && laserMm > 0 && laserMm <= LASER_NEAR_MM) return 2;

if (!isnan(temperatureC) && temperatureC >= HIGH_TEMP_C) return 3;

if (ENABLE_SOIL_ALERT && soilRaw >= SOIL_DRY_THRESHOLD) return 4;

return 0;

}

void updateOutputs() {

digitalWrite(LED_RED, LOW);

digitalWrite(LED_BLUE, LOW);

digitalWrite(LED_GREEN, LOW);

if (activeAlert == 1 || activeAlert == 2) {

digitalWrite(LED_RED, HIGH);

} else if (activeAlert == 3 || activeAlert == 4) {

digitalWrite(LED_BLUE, HIGH);

} else {

digitalWrite(LED_GREEN, HIGH);

}

static unsigned long lastBuzzerToggle = 0;

static bool buzzerOn = false;

if (activeAlert == 0) {

noTone(BUZZER_PIN);

buzzerOn = false;

return;

}

unsigned long interval = 150;

int frequency = 2000;

if (activeAlert == 2) { interval = 300; frequency = 1600; }

if (activeAlert == 3) { interval = 500; frequency = 1000; }

if (activeAlert == 4) { interval = 800; frequency = 700; }

if (millis() - lastBuzzerToggle >= interval) {

lastBuzzerToggle = millis();

buzzerOn = !buzzerOn;

if (buzzerOn) tone(BUZZER_PIN, frequency);

else noTone(BUZZER_PIN);

}

}

// ---------- Individual pages ----------

void pageWelcome() {

drawCenteredText("WELCOME TO", 3, 1);

drawCenteredText("AGRI LINK", 15, 2);

drawSun(108, 20, 5);

drawFarmer(20 + (animationFrame % 75), 34, animationFrame % 2);

display.setTextSize(1);

display.setCursor(79, 39); display.print("SMART");

display.setCursor(79, 48); display.print("FARMING");

}

void pageTemperature() {

drawHeader("TEMPERATURE");

drawSun(20, 29, 7);

display.setTextSize(2);

display.setCursor(42, 19);

if (isnan(temperatureC)) display.print("--.-");

else display.print(temperatureC, 1);

display.setTextSize(1); display.print(" C");

display.setCursor(5, 44); display.print("DHT11 live reading");

drawFooter();

}

void pageHumidity() {

drawHeader("AIR HUMIDITY");

drawWaterDrop(20, 29 + (animationFrame % 3), 5);

display.setTextSize(2);

display.setCursor(43, 20);

if (isnan(humidityPct)) display.print("--");

else display.print(humidityPct, 0);

display.print("%");

display.setTextSize(1);

display.setCursor(5, 44); display.print("Humidity level");

drawFooter();

}

void pageSoil() {

drawHeader("SOIL MOISTURE");

drawPlant(20, 42, 14);

display.setTextSize(2);

display.setCursor(43, 15); display.print(soilRaw);

display.setTextSize(1);

display.setCursor(43, 35); display.print("Raw ADC");

display.setCursor(5, 45);

display.print("Estimate "); display.print(calculateSoilPercent()); display.print("%");

drawFooter();

}

void pageLight() {

drawHeader("LIGHT SENSOR");

drawSun(20 + (animationFrame % 5), 29, 7);

display.setTextSize(2);

display.setCursor(43, 19); display.print(ldrRaw);

display.setTextSize(1);

display.setCursor(43, 39); display.print("LDR raw");

display.setCursor(5, 45);

if (ldrRaw < 1200) display.print("Low light");

else if (ldrRaw < 2800) display.print("Medium light");

else display.print("Bright light");

drawFooter();

}

void pageLaser() {

drawHeader("LASER DISTANCE");

int r = 4 + animationFrame % 9;

display.drawCircle(20, 30, r, SSD1306_WHITE);

display.drawLine(20, 30, 30, 20, SSD1306_WHITE);

display.setTextSize(2);

display.setCursor(42, 19);

if (!laserReady || laserMm == 0) display.print("---");

else display.print(laserMm);

display.setTextSize(1);

display.setCursor(42, 40); display.print("Distance mm");

drawFooter();

}

void pageUltrasonic() {

drawHeader("ULTRASONIC");

int r = 5 + animationFrame % 9;

display.drawCircle(20, 29, 4, SSD1306_WHITE);

display.drawCircle(20, 29, r, SSD1306_WHITE);

display.setTextSize(2);

display.setCursor(42, 19);

if (ultrasonicCm < 0) display.print("---");

else display.print(ultrasonicCm, 1);

display.setTextSize(1);

display.setCursor(42, 40); display.print("Distance cm");

drawFooter();

}

void pageGPS() {

drawHeader("GPS LOCATION");

display.drawCircle(15, 24, 7, SSD1306_WHITE);

display.drawTriangle(10, 29, 20, 29, 15, 39, SSD1306_WHITE);

display.setTextSize(1);

display.setCursor(31, 15);

if (!gpsFix) {

display.print("Searching...");

display.setCursor(5, 43); display.print("Try outdoors");

} else {

display.print("LAT "); display.print(latitude, 4);

display.setCursor(31, 29);

display.print("LON "); display.print(longitude, 4);

}

drawFooter();

}

void pageFarm() {

drawHeader("3D FARM WORLD");

drawSun(110, 19, 4);

drawButterfly(8 + (animationFrame * 2) % 108,

18 + animationFrame % 3,

animationFrame % 2 == 0);

drawPerspectiveField();

drawTree(8, 40);

drawTree(120, 40);

int animalX = 14 + animationFrame % 92;

if (animationFrame % 2 == 0) drawChicken(animalX, 40);

else drawCow(animalX, 42);

drawFarmer(35 + animationFrame % 55, 20, animationFrame % 2);

drawFooter();

}

void pageAlert() {

if (animationFrame % 2 == 0) {

display.drawRect(0, 0, 128, 64, SSD1306_WHITE);

display.drawRect(2, 2, 124, 60, SSD1306_WHITE);

}

drawCenteredText("FIELD ALERT!", 5, 1);

int r = 8 + animationFrame % 5;

display.drawTriangle(64, 19-r, 64-r, 20+r, 64+r, 20+r, SSD1306_WHITE);

display.setTextSize(1);

display.setCursor(62, 18); display.print("!");

if (activeAlert == 1) drawCenteredText("OBJECT VERY NEAR", 42, 1);

if (activeAlert == 2) drawCenteredText("LASER OBJECT NEAR", 42, 1);

if (activeAlert == 3) drawCenteredText("HIGH TEMPERATURE", 42, 1);

if (activeAlert == 4) drawCenteredText("CHECK SOIL MOISTURE", 42, 1);

drawCenteredText("CHECK YOUR FIELD", 54, 1);

}

void renderScreen() {

display.clearDisplay();

if (activeAlert != 0) {

pageAlert();

} else {

switch (currentPage) {

case 0: pageWelcome(); break;

case 1: pageTemperature(); break;

case 2: pageHumidity(); break;

case 3: pageSoil(); break;

case 4: pageLight(); break;

case 5: pageLaser(); break;

case 6: pageUltrasonic(); break;

case 7: pageGPS(); break;

case 8: pageFarm(); break;

}

}

display.display();

}

// ---------- Setup ----------

void setup() {

Serial.begin(115200);

pinMode(TRIG_PIN, OUTPUT);

pinMode(ECHO_PIN, INPUT);

pinMode(BUZZER_PIN, OUTPUT);

pinMode(LED_RED, OUTPUT);

pinMode(LED_BLUE, OUTPUT);

pinMode(LED_GREEN, OUTPUT);

digitalWrite(TRIG_PIN, LOW);

analogReadResolution(12);

dht.begin();

GPS_Serial.begin(9600, SERIAL_8N1, GPS_RX, GPS_TX);

Wire.begin(21, 22);

if (!display.begin(SSD1306_SWITCHCAPVCC)) {

Serial.println("OLED initialization failed. Check wiring.");

while (true) delay(100);

}

display.clearDisplay();

display.display();

laser.setTimeout(100);

laserReady = laser.init();

if (laserReady) {

laser.startContinuous(100);

Serial.println("VL53L0X ready");

} else {

Serial.println("VL53L0X not detected; other pages will continue");

}

// Startup welcome animation

for (int i = 0; i < 12; i++) {

display.clearDisplay();

drawCenteredText("AGRI LINK", 10, 2);

drawCenteredText("SMART AGRICULTURE", 34, 1);

drawFarmer(15 + i * 8, 20, i % 2);

display.display();

delay(100);

}

lastPageChange = millis();

lastFrame = millis();

}

// ---------- Main loop ----------

void loop() {

unsigned long now = millis();

readSensors();

activeAlert = determineAlert();

updateOutputs();

// Advance pages every 5 seconds when no alert is active.

if (activeAlert == 0 && now - lastPageChange >= PAGE_INTERVAL_MS) {

lastPageChange = now;

currentPage = (currentPage + 1) % PAGE_COUNT;

}

// Animate at about 10 frames per second.

if (now - lastFrame >= FRAME_INTERVAL_MS) {

lastFrame = now;

animationFrame++;

renderScreen();

}

}